Department of Electrical Engineering

| Faculty Member: Ma’am Qurat-ul-ain | Dated: December 12, 2020 |
| --- | --- |
| Course/Section: BSCS-9B | Semester: 3rd |

Computer Organization and

Assembly Language (CS235)

Lab #8 Bit wise and Shift operations in Assembly Language

|  |  | PLO4 |  | PLO5 | PLO8 | PLO9 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Roll number | Viva /Quiz/ Lab performance 5 marks | Analysis of data in lab report 5 marks | Modern tool Usage 5 marks | Ethics and Safety 5 marks | Individual and team work 5 marks | Total 25 marks |
| Fatima Seemab | 291310 |  |  |  |  |  |  |
| Mahum Samar | 290647 |  |  |  |  |  |  |
| Maryam Fatima | 290479 |  |  |  |  |  |  |

Objective: The objective of this lab is to use bitwise operator for multiplication in Assembly language

Mask: A mask is a bit pattern that is defined by a programmer, which allows specific bits in a piece of data to be set, cleared, tested or altered.

Setting Bits to 1:

To turn on a specific bit, we can use the OR bitwise operation and a suitable mask.

For example, if we need to turn on Bit 4 and Bit 7 of a byte (remember that the bit on the right hand side is Bit 0), you can use the mask 1001 0000 and the OR bitwise operation.

Resetting/Clear bits to 0
We can't force a bit to be 0 using the OR command. We can use the bitwise command AND with a suitable mask,

For example, we want to reset Bits 0, 1 and 2 in a byte but leave all the other bits as they were. You would use the mask 1111 1000 along with the AND bitwise operator.

;

The TEST instruction

The test instruction performs a bitwise AND on two operands. The flags SF, ZF, PF are modified while the result of the AND is discarded.

- TEST destination, source

  - Performs AND, does not store the result

  - Flags are set

Q#1 Write assembly code to find number of ones in32-bit hex number using bit wise operation.

Code:

TITLE Number of Ones in 32-bit Hex Number

INCLUDE irvine32.inc

.data

sInput BYTE "Enter a 32-bit Hex Number: ", 0

sOutput BYTE "Number of ones in 32-bit Hex Number are ",0

.code

main proc

mov edx, offset sInput

call WriteString	; prompt for input

call ReadHex

mov ebx,eax

call NumberOfOnes	; method call

exit

main endp

NumberOfOnes proc uses ebx

;--------------------------------------

; Receives 32-bit hex number in ebx

; Returns number of ones in eax

;--------------------------------------

mov eax,0		; initialize ecx

NotZero:

shr ebx,1	; shift right and check CF

jnc L		; if carry flag is not 1 jump

inc al		; else increment eax

L:

cmp ebx,0	; loop until ebx becomes 0

jnz NotZero

mov edx, offset sOutput

call WriteString

call WriteDec

call crlf

ret

NumberOfOnes endp

end main

Output:

Q#2 Write equivalent assembly code of given C++ program

void main(){

Int a,b,result;

printf("nEnter the numbers to be multiplied :");

scanf("%d%d",&a,&b);         // Provide input such that a>b

result=0; // if b>a , result is 0

while(b != 0)               // Iterate the loop till b==0

{

if (b&01)                // Bitwise & of the value of b with 01

{

result=result+a;     // Add a to result if b is odd .

}

a<<=1;                   // Left shifting the value contained in 'a' by 1

// multiplies a by 2 for each loop

b>>=1;                   // Right shifting the value contained in 'b' by 1.

}

printf("nResult:%d",result); //display result using writeint comand

}

Code:

TITLE Multiply 2 Numbers

INCLUDE irvine32.inc

.data

a DWORD ?

b DWORD ?

result DWORD 0

sInput BYTE "Enter the numbers to be multiplied: ", 13, 10, 0

sOutput BYTE "Result: ", 0

.code

main proc

mov edx, offset sInput

call WriteString		; Prompt user

call ReadInt			; Read 1st integer

mov a, eax

call ReadInt			; Read 2nd integer

mov b, eax

call Mult

exit

main endp

Mult proc uses eax

;-------------------------------------------------

; Receives the numbers to be multiplied in a and b

; Returns answer in 'result'

;-------------------------------------------------

; to be used for storing result as

; add instruction cannot have memory

; location as both destination and source

mov eax, 0

cmp b, 0

jz L				; Do not loop if b = 0

bNotZero:

test b, 01

jz bNotOdd

add eax, a

bNotOdd:

shl a, 1	; Left shifting the value in 'a' by 1

shr b, 1	; Right shifting the value in 'b' by 1

cmp b, 0

jnz bNotZero			; Iterate the loop till b = 0

L:

mov edx, offset sOutput

call WriteString		; Display output

call WriteInt

; move eax to result

mov result, eax		; Move eax to result

call crlf

ret

Mult endp

end main

Output:

Q#3 Write assembly code to take a string from user and convert the string into all capital.

(Print your name, do not use the spaces)

Code:

TITLE Convert the String into all CAPITAL

INCLUDE irvine32.inc

.data

string BYTE ?

sInput BYTE "Enter a string without space to convert to UPPERCASE: ",13,10,9,0

sOutput BYTE "In uppercase: ",13,10,9,0

;----------------------------------------------------

; 13 = carriage return

; 10 = line field

;  9 = horizontal tab

;----------------------------------------------------

.code

main proc

mov edx, offset sInput		; prompt user

call WriteString

mov edx, offset string		; read string

mov ecx, 64				; maximum number of characters + 1 for string-end indicator

call readString				; returns length of entered string in al

; input string should not contain space

; because AND-ing mask 11011111b

; with 00100000b (ascii for space) will

; result into 00000000b i.e. NULL

; the end-string indicator

mov ecx, lengthof string

call allCapital			; method call

mov edx, offset sOutput

call WriteString			; display output

mov edx, offset string

call WriteString

call crlf

exit

main endp

allCapital proc uses edx

;----------------------------------------------------

; Recieves edx = offset of 'string' variable

;	    al = Number of characters entered by user

; Returns string in upper case in same variable

;----------------------------------------------------

movzx ecx, al

; 11011111b is the MASK to turn caps lock on

L:

and BYTE PTR [edx], 11011111b

inc edx

loop L

ret

allCapital endp

end main

Output: